DIESEL ENGINE SOUND LEVEL INTENSITY COMPARISON

| dB=10logSoundIntensityAmbientSoundIntensity Deutz Vs Cummins Sound Level Intensity |  |  |  |
| --- | --- | --- | --- |
|  | Cummins | Deutz | Deutz Vs. Cummins Sound Level Intensity |
| Max Idle RPM (dB) | 98 | 106 | Deutz 6X Cummins Sound Intensity |
| Max Full RPM (dB) | 108 | 122 | Deutz 25X Cummins Sound Intensity |

| OSHA 1926.52 Table D-2 Permissible Noise Exposures |  |
| --- | --- |
| Duration per Day (Hours) | Sound Level (dBA) Slow Response |
| 8 | 90 |
| 6 | 92 |
| 4 | 95 |
| 3 | 97 |
| 2 | 100 |
| 1 1/2 | 102 |
| 1 | 105 |
| 1 1/2 | 110 |
| 1/4 or Less | 115 |

| Diesel Engine Sound Level 1 Meter Distance |  |  |
| --- | --- | --- |
|  | Cummins QSB6.7 | Deutz BF914 |
| Idle RPM (dB) | 96 - 98 | 102 - 106 |
| Full RPM (dB) | 104 - 108 | 119 - 122 |

Decibel (dB) is the logarithmic unit of measure of the ratio between 2 sound intensities;

Using the decibel scale, the sound threshold (approximately total silence) is 0 dB. Sound 10 times more powerful is 10 dB. Sound 100 times more powerful is 20 dB. Sound 1000 times more powerful than near total silence is 30 dB. Following are approximate decibel ratings for common sounds:

Perceived Total Silence	    0 dB		Jackhammer			100 dB

Whisper			  15 dB 		Car Horn			110 dB

Normal Conversation	  60 dB		Rock Concerts 		120 dB

Lawnmower			  90 dB 		Pain Threshold		130 dB