- HYDRAULIC AND ELECTRICAL SCHEMATICS

Cummins Error Codes

Code & descriptions

EC01 – No radio link (no signal from either radio or cable)

EC02 – J1939 CAN link error

EC03 – RPM Plus

EC04 – RPM Minus

EC05 – Volume Plus

EC06 – Volume Minus

EC07 – Horn

EC08 – Vibrator

EC09 – Safety Valve 1

EC10 – Safety Valve 2

EC11 – Pump Coil A

EC12 – Pump Coil B

EC13 – S Tube A

EC14 – S Tube B

EC15 – Reverse Coil Output – enabled when calibration checkbox for Reverse Logic Change is unchecked.

EC16 – Fast Change – enabled when the calibration checkbox for Fast Change is checked.

EC17 – Reverse Light out Error

EC18 – Pump Light Out Error

EC19 – Check Engine Light

EC20 – Stop Engine Light

EC21 – Volume PWM Output Error

EC22 – Prox A&B Error

EC23 – Wrong ID

EC24 – Engine RPM Overspeed

EC25 – Oil Low Pressure

EC26 – Coolant Over Temp.

EC27 – Low Batt. Voltage

EC28 – High Batt. Voltage

EC29 – Denison Fast Change

EC30 – Denison A (only on C series with Denison Hydraulic pumps)

EC31 – Denison B (only on C series with Denison Hydraulic pumps)

START HERE

TRANSMITTER RF. I.D

TEACH PRODUCE

STEP 1

TURN OFF RADIO FIRST BY PUSHING AND HOLDING THE POWER BUTTON ON THE TOP OF THE TRANSMITTER.

NOTE: IF THE TRANSMITTER IS STILL ON

THE GREEN LED WILL LIGHT WHEN ANY SWITCH OTHER THAN THE POWER BUTTON IS PRESSED.

TRANSMITTER

CONTROL BOX

STEP 2

PRESS AND HOLD PUMP ON, RPM+, STROKE CHANGE, AND REVERSE SIMULATANEOSLY. WHILE HOLDING THESE FOUR SWITCHES, TURN THE RADIO ON BY PRESSING THE POWER BUTTON.

THEN RELEASE ALL SWITCH

RETURN PUMP ON/OFF SWITCH TO OFF AND FWD/REV SWITCH TO FORWARD.

THE RED AND GREEN LED’S SHOULD FLASH ALTERNATLY.

RADIO REMOTE PROGRAMING INSTRUCTIONS

CONTROL BOX SET-UP

STEP 3

TURN ON IGNITION KEY

NOTE: ENGINE DOES NOT NEED TO BE STARTED

STEP 4

THE TWO LED’S ON THE TRANSMITTER WILL STOP ALTERNATELY FLASHING. WILL COME ON STEADY TOGETHER FOR A FEW SECONDS AND THEN GO OFF WHEN THE TRANSMITTER AND RECIEVER ARE CORRECTLY SET UP FOR COMMUNICATION.

SET THE CONTROL BOX TO “REMOTE” AND IF THE CONTROL BOX DOES NOT SUCCESSFULLY RECEIVE COMMANDS FROM THE TRANSMITTER.

REPEAT THE PROCEDURE FROM STEP 1.